SELF-AUDIT QUESTIONNAIRE (FTH)

The purpose of this form is to assist the KOREAN AIR in performing a document audit evaluation of full technical handling line maintenance services. Failure to provide the requested information may result in further in-depth screening actions, such as video interviews and on-site audits.

Please fill in the information below.

| Line Station | HKG |
| --- | --- |
| Company Name | Hong Kong Aircraft Engineeing Company Limited |
| Audit Date (DD-MMM-YYYY) | 25 Sep 2026 |
| Auditee Name | Alan Choi |
| Auditee Title/Department | Assistant Manager, Line Maintenance Support |

I. Section One

Write an answer and give a brief explanation in the comments section. Please provide supporting evidence material if needed (save the file name to begin with the number corresponding to the relevant question. For example, “1-1 Facility Change” and “3-5 Quality Auditors Requirement/Qualification”).

| 1. Facility |  |
| --- | --- |
| 1-1 | Have there been any changes with the maintenance facilities, such as maintenance office/parts warehouse/tool room/hangar in the last 2 years? (YES/NO) If yes, provide any written documentation that would support your answer. |
| Comments: No. There have been no changes to the maintenance facilities, including maintenance offices, parts warehouse, tool room, or hangar facilities, within the last two years. |  |
| 2. Management / Personnel |  |
| 2-1 | Do you have sufficient personnel to perform the Line Maintenance work contracted by KE? (YES/NO) If you have any other station personnel frequently deployed to replace the maintenance personnel within the last 3 months, please provide details and reasons. |
| Comments: Yes. HAECO has sufficient qualified and authorized personnel to perform the line maintenance work contracted by Korean Air (KE). There are approximately 60 B1 certifying engineers available to support line maintenance operations at Hong Kong Station. |  |
| 2-2 | Are all facilities, equipment, tools, materials, technical data, and certified mechanics required to perform authorized maintenance work available? (YES/NO) |
| Comments: Yes. All required facilities, equipment, tools, materials, current technical data, and qualified certifying personnel necessary to perform authorized maintenance work are available and maintained in accordance with HAECO procedures and regulatory requirements. |  |
| 2-3 | Does the AMO certificate include the following? (YES/NO) - The actual status of the facility matches the maintenance regulations such as AMOPM, RSQCM, MOM, etc. - Authorization date and validity period - Authorized scope of work |
| Comments: Yes, AMO is as attached |  |
| 2-4 | Who is responsible for supervising and managing a maintenance organization approved by the aviation authority? |
| Comments: HAECO Hong Kong CEO |  |
| 2-5 | Who is responsible for ensuring compliance with the maintenance organization's accreditation requirements? |
| Comments: Quality Manager of HAECO Hong Kong |  |
| 3. Quality Assurance |  |
| 3-1 | Does the AMOPM include the following requirements? (Please provide the AMOPM and indicate the page numbers of that document where relevant information can be referenced for the items below) (i) A brief description of the organization that includes: (a) A general description of the scope of work authorized under the organization's terms of approval; (PAGE : 11-14 ) (b) A general description of the organization's facilities. (PAGE : 13-14 ) (ii) A description of the organization procedures and quality or inspection system; (PAGE : 7, 9-10 and 17 ) (iii) Names and duties of the responsible personnel; (PAGE : 10 ) (iv) Names and duties of the person or persons whose responsibilities are to ensure that maintenance is carried out in accordance with the AMOPM, RSQCM, MOM, etc.; (PAGE : 18 ) (v) A description of the procedures used to establish the competence of maintenance personnel; (PAGE : 14 and 18 ) (vi) A description of the methods used for the completion and retention of the Operator's maintenance records, including procedures for retaining backup records; (PAGE : 20 ) (vii) A description of the procedure for preparing the maintenance release and the circumstances under which the release is to be signed; (PAGE : 18 ) (viii) The process for authorizing personnel to sign the maintenance release and the scope of their authorization; (PAGE : 18 ) (ix) A description of any additional procedures for complying with the Operator's maintenance procedures and requirements; (PAGE : 20 ) (x) A description of the procedures for complying with the service information reporting requirements; (PAGE : 19 ) (xi) A description of the procedure for receiving, amending and distributing within the maintenance organization, all necessary airworthiness data from the type certificate holder or aircraft Type Design Organization (PAGE : 17 ) |
| 3-2 | Is AMOPM revised to reflect the latest situation, and is the revised content promptly provided to all organizations/personnel to which the manual was issued? (YES/NO) |
| Comments: Yes. The AMOPM is revised as necessary to reflect the latest organizational and regulatory requirements. Following approval, the updated version is distributed to relevant department heads through Coruson for reference and implementation. |  |
| 3-3 | Are internal audits being accomplished as scheduled and discrepancies analyzed to determine the cause not just the symptom and corrected in a timely manner? (YES/NO) |
| Comments: Yes. Internal audits are conducted in accordance with the approved audit programme by HAECO Quality Assurance. Audit findings are reviewed, root causes are analyzed, and corrective actions are implemented and monitored to ensure timely closure. |  |
| 3-4 | Do you periodically review the suitability of the quality assurance program applied within your organization? (YES/NO) |
| Comments: Yes. The suitability and effectiveness of the Quality Assurance Programme are reviewed on a regular basis through Safety Action Group meetings, management reviews, audit results, and quality performance monitoring. |  |
| 3-5 | Is there a training and qualification management program for quality auditors? (YES/NO) |
| Comments: Yes. HAECO maintains a training and competency management programme for quality auditors, including qualification requirements, competency assessments, and recurrent training where applicable. |  |
| 4. Training/Qualification |  |
| 4-1 | Does the roster of certifying staffs match the personnel information within your organization? (Changes must be reflected in the list within 5 days) (YES/NO) |
| Comments: Yes |  |
| 4-2 | Are recurring training requirements for Human Factors training being tracked, performed and documented? ( YES / NO ) |
| Comments: Yes, Initial Human Factors training is provided under course T.F241, and recurrent Human Factors training is conducted under course T.F162 at a two-year interval. |  |
| 5. Maintenance Records |  |
| 5-1 | In addition to the aircraft types that support KE, please also describe the aircraft types that support line maintenance work for other customers. |
| Comments: In addition to supporting Korean Air (KE) operations, HAECO is approved and authorized to provide line maintenance services for other customer airlines on the following aircraft types within the approved scope of work: - Airbus A320/A321 family (V2500, LEAP-1A and PW1100 engines) - Airbus A330-200/-300 - Airbus A350-900 - Airbus A380-800 - Boeing 737NG family (-600/-700/-800/-900) - Boeing 747-400 and 747-8 - Boeing 777 family (B777-200/-300/-300ER) - Boeing 787 family (B787-8/-9/-10) |  |
| 5-2 | How long will maintenance records be kept and photo evidence of protection against damage/fire/theft? |
| Comments: A copy of maintenance record is retained by HAECO for a minimum of 2 years from the date of the release. |  |
| 6. Technical Data |  |
| 6-1 | Please describe how to access the manual when TDMS is unavailable. |
| Comments: According to procedure highlighted in TPC26-0019 e-form |  |
| 6-2 | Please describe how your company properly manages and applies the Technical Information and Quality Information issued by KE. |
| Comments: When TPC (Technical publication control) received KE technical data, TPC will download and upload into our HAECO ETMS (Technical Manual platform for production) and acknowledge receipt if required. |  |
| 6-3 | Do you have and use a ‘Reference Only’ stamp or something similar for making copies of technical documentation? ( YES / NO ) |
| Comments: TPC do not use Reference Only Stamp.. it should be applicable to production staff who may print out from PC. In general, staff should refer to GTI-P-188 (Technical/Maintenance Data) para. 13.2 and destroy after use. |  |
| 7. Tool/Equipment |  |
| 7-1 | Are the tools on the ramp car properly identified and marked with the correct information (without the risk of damage or loss)? (YES/NO) |
| Comments: Yes. All tools are uniquely identified and labelled with the appropriate tool identification information, including part number and serial number where applicable, to ensure traceability and prevent loss or misidentification. |  |
| 7-2 | How do you demonstrate that the equipment, tools, or materials being replaced are at least as good as those recommended by the manufacturer? |
| Comments: Maintenance personnel are required to perform a pre-use inspection of tools and equipment before use to ensure they are in good condition, serviceable, and suitable for the intended maintenance task. Any unserviceable tools or equipment are removed from service and reported in accordance with HAECO procedures. |  |
| 7-3 | Are procedures related to equipment/tools FOD prevention followed? (YES/NO) |
| Comments: Yes. Procedures for tool control and Foreign Object Debris (FOD) prevention are implemented and followed in accordance with HAECO procedure GTI-P-148 FOD Prevention. |  |
| 7-4 | Is there a manufacturer's manual for the equipment/tools used for maintenance work or is it available for viewing? (YES/NO) |
| Comments: Yes. Applicable manufacturer's maintenance manuals, equipment manuals, operating instructions, and technical data are available to maintenance personnel and can be accessed when performing maintenance activities. |  |
| 7-5 | If you have a list of assigned tools/equipment provided by KE, does it match the actual product and equipment name, serial number, and quantity? (YES/NO) |
| Comments: Awaiting MM feedback |  |
| 8. Material |  |
| 8-1 | Please check if the allocation parts stored in the station have not exceeded the preservation due date and let us know the results. |
| Comments: Awaiting MM feedback |  |
| 8-2 | Is there anything that has passed the expiration date on packing, O-ring, tape, RTV, battery, etc.? (YES/NO) |
| Comments: Awaiting MM feedback |  |
| 8-3 | After checking the items with shortened expiration dates due to opening, are there any items whose expiration dates have passed? (YES/NO) |
| Comments: Awaiting MM feedback |  |
| 8-4 | How do mechanics manage ESDS materials according to relevant manuals and procedures and learn how to use Wrist Straps? |
| Comments: There is no ESDS materials used in Line Maintenance |  |
| 8-5 | Are flammable or toxic chemicals stored separately from general materials? (YES/NO) |
| Comments: Awaiting MM feedback |  |

II. Section Two

Please collect documents and photos supporting the information below:

- For photos, replace the reference images with the relevant photo.

- For documents, prepare the file with an ID matching the question number added to the beginning of the file name.

Should you have trouble sending this word file due to the large file size of images, please collect all data in a separate compressed file/folder with an ID matching the question number added to the beginning of the file name. For example, “B-3 AMO Certificates” and “B-4 MOPMs”.